# A No-Wait Single-Intersection Scheduler by Priority Enumeration: A Decision Tree with a Longest-Path Timing Model

# Problem Formulation

Let N={1,…,N} index the vehicles to be scheduled through the merging zone, and let M be the set of conflict points (spaces). Vehicle n traverses an ordered sequence of sub-tasks s∈Sn={1,…,Sn}; sub-task n,s occupies conflict point σn,s∈M for a fixed duration cn,s>0. Define the prefix offset and the total traversal length

πn,s=k=1s−1cn,k,  Cn=s∈Sn​cn,s,

and let αn denote the earliest admissible entry (release) time of vehicle n.

## No-wait motion

Each vehicle crosses the intersection without stopping, so its sub-tasks are contiguous. The schedule of vehicle n is therefore parameterized by a single entry time tn≥αn, under which sub-task n,s occupies σn,s over the interval

In,stn= tn+πn,s,  tn+πn,s+cn,s .

## Scheduling problem

Two sub-tasks conflict iff they use the same conflict point; collect such pairs in

K= {n,s,n′,s′}:σn,s=σn′,s′, n≠n′ .

A schedule is feasible iff conflicting occupations never overlap. Minimizing the total delay yields

P  mint∈RN n∈N​tn−αn

subject to

tn≥αn, n∈N,

In,stn∩In′,s′tn′=⌀, ∀{n,s,n′,s′}∈K.

Since motion is no-wait, the exit time of n is tn+Cn, so n​tn−αn equals the total completion delay.

# Disjunctive Form and the Selection

Each non-overlap constraint is a disjunction. For {n,s,n′,s′}∈K, the orientation “n,s before n′,s′” reads

tn′−tn ≥ wn→n′ := πn,s+cn,s−πn′,s′,

and the reverse orientation has weight wn′→n=πn′,s′+cn′,s′−πn,s. A selection ξ orients every conflict pair, i.e. it is a set of weighted precedence arcs i→wj on the vehicle nodes. Adjoining the release arcs 0→αnn from a source node 0 yields the precedence graph Gξ={0}∪N, Aξ.

# Timing Model

Proposition 1 (Earliest-entry timing). If Gξ is acyclic, then problem P restricted to the orientation ξ has a unique pointwise-minimal solution t⋆ξ, characterized by the longest-path recursion

tn⋆ξ=maxαn, max i: i→n∈Aξti⋆ξ+wi→n, n∈N,

computed by the fixed-point (Bellman–Ford) iteration EarliestTξ. If Gξ contains a positive cycle, ξ is infeasible.

The recursion above is the timing model EarliestT:ξ↦t⋆ξ: it maps an ordering to the unique contention-free, no-wait schedule that starts every vehicle as early as the chosen precedences permit. Its cost is

Jξ = n∈N​tn⋆ξ−αn.

# Reduction to a Finite Search

Proposition 2. minP=minξ∈ΞJξ, where Ξ is the finite set of acyclic selections.

The optimum is attained by a semi-active schedule; hence it suffices to search over orderings, the timing model supplying the optimal entry times of each.

# Decision-Tree Search

A node of the search tree is a partial selection ξ with decided set D⊆K. The search interleaves the timing model with conflict detection. The conflict oracle FindConflictt,D returns the first pair {n,s,n′,s′}∈K\D whose intervals overlap at the current schedule t, i.e. satisfying

tn+πn,s<tn′+πn′,s′+cn′,s′  ∧  tn′+πn′,s′<tn+πn,s+cn,s,

and returns $\textsc{none}$ if no such pair exists.

Algorithm 1: No-wait enumeration scheduler. Recurseξ,D:

- t←EarliestTξ    (timing model, Prop. 1)

- if $t=\textsc{none}$: return    (positive cycle ⇒ infeasible — prune)

- κ←FindConflictt,D    (first overlapping, undecided pair)

- if $\kappa=\textsc{none}$: record leaf t⋆←t; return    (conflict-free complete schedule)

- {n,s,n′,s′}←κ

- Recurse​ξ∪{n→wn→n′n′}, D∪{κ}    (branch: n,s before n′,s′)

- Recurse​ξ∪{n′→wn′→nn}, D∪{κ}    (branch: n′,s′ before n,s)

Initialize with Recurse⌀,⌀.

The set of leaves enumerates {t⋆ξ:ξ∈Ξ}, i.e. all contention-free complete schedules; the minimum-cost leaf solves P. A positive-cycle test in EarliestT prunes infeasible orientations, so only acyclic selections are expanded.